HYDROLOGICAL AND HYDRAULIC MODELING

AI/ML-Based Flood Prediction System

End-to-End Workflow Documentation

Complete Technical Process Flow

From Data Collection to Actionable Insights

© 2026 Straive. All rights reserved.

# Table of Contents

- System Overview

- Phase 1: Data Inputs & Collection

- Phase 2: Physical Modeling & Simulation

- Phase 3: AI/ML Enhancement (Future)

- Phase 4: Outputs & Visualizations

- Data Flow Architecture

- Integration Points & APIs

- Appendix: Technical Specifications

# 1. System Overview

The Hydrological and Hydraulic Modeling + AI/ML-Based Prediction system is a comprehensive flood forecasting and early warning platform that integrates real-time sensor data, geospatial analysis, physical modeling, and predictive intelligence to provide actionable flood risk assessments and emergency response guidance.

## 1.1 System Architecture

The system operates through four integrated pillars:

- Data Inputs: Real-time sensor networks, satellite imagery, and geospatial datasets

- Physical Modeling: Hydrological and hydraulic simulation engines

- AI/ML Enhancement: Predictive models for forecasting and susceptibility mapping (Phase 2)

- Outputs & Alerts: Flood maps, exposure analysis, and early warning notifications

## 1.2 Workflow Summary

Input → Process → Model → Enhance → Output → Act

# 2. Phase 1: Data Inputs & Collection

Data inputs form the foundation of the flood prediction system. Multiple data sources are ingested, normalized, and validated before being fed into physical models.

## 2.1 River Water Level Sensors

| Component | What It Generates | Measurement Range | Classification |
| --- | --- | --- | --- |
| River Water Level Sensors | Real-time river stage levels | 0.1–30+ m | Normal <3m, Warning 3–5m, Danger >5m |

Process Flow:

- Sensors installed at strategic river gauging stations

- Data transmitted via IoT/cellular networks to central server

- API integration: Sensor data ingested through RESTful API endpoints

- Time-series normalization: Data standardized to 5-minute intervals

- Validation: Outlier detection and quality control algorithms applied

- Storage: Time-series database (InfluxDB/TimescaleDB) for historical analysis

Data Format:

JSON API Response Example:

{ "sensor_id": "RWL_001", "timestamp": "2026-05-18T10:30:00Z", "water_level_m": 4.2, "location": {"lat": 11.0168, "lon": 76.9558}, "status": "warning" }

Integration Requirement:

Physical modeling engines (HEC-RAS, SWMM) require water level inputs as boundary conditions for hydraulic simulations.

## 2.2 Rainfall Gauges (Rain Intensity)

| Component | What It Generates | Measurement Range | Classification |
| --- | --- | --- | --- |
| Rainfall Gauges | Rainfall intensity & accumulation | 0–300 mm/hr | Low <20, Moderate 20–50, Extreme >100 |

Process Flow:

- Tipping bucket rain gauges deployed across watershed

- Spatial interpolation using kriging/IDW for ungauged areas

- Satellite precipitation products (GPM, TRMM) integrated for coverage gaps

- Cumulative rainfall calculated over sliding windows (1hr, 6hr, 24hr)

Integration Requirement:

Rainfall data feeds into Rainfall-Runoff models to calculate watershed discharge and trigger threshold analysis for landslide/flood risk.

# 3. Phase 2: Physical Modeling & Simulation

Physical models simulate the hydrological and hydraulic processes that govern flood behavior. These deterministic models use conservation equations (mass, momentum, energy) to predict water flow and inundation.

## 3.1 Rainfall-Runoff & Watershed Hydrology

| Model Component | Model Output | Output Range | Usage |
| --- | --- | --- | --- |
| Rainfall-Runoff & Watershed Hydrology | Basin discharge volume, flow rates & timing | 1–100,000 m³/s | Boundary condition for river hydraulics |

Model Details:

- Software: HEC-HMS (Hydrologic Engineering Center - Hydrologic Modeling System)

- Method: SCS Curve Number / Green-Ampt infiltration models

- Input Required: Rainfall time series, watershed boundaries, soil types, land use/cover, DEM

- Process: Converts rainfall excess to runoff using unit hydrograph theory

Workflow Steps:

- Watershed Delineation: Use DEM to identify catchment boundaries and flow paths

- Curve Number Assignment: Based on soil hydrologic group and land use

- Rainfall Distribution: Apply temporal pattern (e.g., SCS Type II storm)

- Runoff Calculation: Compute direct runoff hydrograph

- Routing: Route flows through channel network to outlet

Output Format:

Discharge hydrograph (m³/s vs. time) exported as CSV for downstream models.

## 3.2 River Hydraulics & Inundation Modeling

| Model Component | Model Output | Output Range | Usage |
| --- | --- | --- | --- |
| River Hydraulics & Inundation Modeling | 2D/3D flow simulation, flood extent & depth mapping | 0–15 m depth, velocity 0–10 m/s | Flood hazard mapping, evacuation planning |

Model Details:

- Software: HEC-RAS (River Analysis System) with 2D extension

- Governing Equations: 2D Saint-Venant equations (shallow water equations)

- Input Required: DEM, river cross-sections, discharge hydrograph from HEC-HMS, Manning's roughness

- Mesh Resolution: 5–30 m computational cells depending on terrain complexity

Workflow Steps:

- Terrain Preparation: Process DEM to create 2D flow area mesh

- Boundary Conditions: Apply discharge hydrograph at upstream boundary

- Manning's n Assignment: Roughness coefficients based on land cover (0.03–0.15)

- Simulation: Solve 2D equations using implicit finite volume scheme

- Post-Processing: Extract depth grids, velocity grids, inundation polygons

Output Format:

- GeoTIFF raster: Flood depth (meters) at each grid cell

- Shapefile polygons: Inundation extent boundaries

- Velocity magnitude rasters for hazard classification

# 4. Phase 3: AI/ML Enhancement (Future Roadmap)

Phase 2 integration (planned for 3+ months post sensor data availability) will augment deterministic physical models with predictive intelligence using machine learning algorithms trained on historical flood events.

## 4.1 LSTM Neural Networks for Flood Forecasting

Model Purpose:

Long Short-Term Memory (LSTM) networks excel at learning temporal patterns in sequential data, making them ideal for time-series flood prediction based on rainfall-runoff relationships.

Training Data Required:

- 3+ years of hourly rainfall, river level, and discharge data

- Historical flood event records with peak flow measurements

- Antecedent moisture conditions (soil saturation)

- Meteorological forecasts (precipitation, temperature)

Model Architecture:

- Input Layer: 72-hour lookback window (rainfall, water level, soil moisture)

- Hidden Layers: 2–3 LSTM layers with 128–256 units each

- Output Layer: 24-hour ahead flood stage prediction

- Loss Function: Mean Squared Error (MSE) for regression

Output Format:

Probabilistic flood stage forecast with confidence intervals (m) at 1-hour intervals for the next 24 hours.

Integration Point:

LSTM predictions can be fused with physical model outputs using ensemble methods (weighted averaging, Bayesian Model Averaging) to improve forecast accuracy and provide uncertainty quantification.

## 4.2 Random Forest Flood Susceptibility Mapping

Model Purpose:

Classify areas as high/medium/low flood susceptibility based on geospatial features (elevation, slope, distance to river, land use, soil type).

Training Data:

- Historical flood extent polygons from satellite imagery (Sentinel, Landsat)

- Topographic derivatives (slope, aspect, curvature, TWI)

- Proximity features (distance to rivers, drainage density)

- Land cover classification, NDVI (vegetation index)

Output Format:

GeoTIFF raster map with pixel values 0–1 (susceptibility score). Classify into Low (<0.3), Medium (0.3–0.6), High (>0.6) risk zones.

# 5. Phase 4: Outputs & Visualizations

Model outputs are transformed into actionable decision-support products for emergency responders, government agencies, and affected communities.

## 5.1 Flood Extent & Inundation Map

| Output Product | What It Shows | Operational Range | Severity Classification |
| --- | --- | --- | --- |
| Flood Extent & Inundation Map | Flooded area polygons | <5 to 10,000+ sq km | Minor / Moderate / Severe |

Visualization Method:

- Web GIS Platform: ArcGIS Online / Mapbox GL JS

- Base Layer: OpenStreetMap or satellite imagery for context

- Flood Depth Symbology: Blue color ramp (light blue <0.5m, dark blue 2–5m, red >5m)

- Interactive Features: Click on polygon to view depth statistics, affected area

Export Formats:

- Shapefile / GeoJSON for GIS integration

- KML/KMZ for Google Earth visualization

- Static PNG/PDF maps for emergency briefings

## 5.2 Population & Infrastructure Exposure Analysis

Process:

- Spatial Overlay: Intersect flood extent polygons with population density grid (WorldPop, 100m resolution)

- Building Footprint Analysis: Overlay with OpenStreetMap buildings, identify affected structures

- Infrastructure Assessment: Count roads, bridges, hospitals, schools within flood zone

Output Dashboard:

- Total affected population: 150,000 people (Low / Medium / Critical)

- Residential buildings: 12,500 structures

- Critical infrastructure: 8 hospitals, 45 schools, 120 km roads

- Economic Loss Estimate: ₹2,500 crore (based on asset valuation)

## 5.3 Early Warning Notifications

Alert Trigger Thresholds:

- Advisory: Water level 70% of danger level, 12–24 hr lead time

- Watch: Water level 85% of danger level, 6–12 hr lead time

- Warning: Water level 95% of danger level, 3–6 hr lead time

- Emergency: Danger level exceeded, immediate action required

Dissemination Channels:

- SMS/WhatsApp: Targeted messages to registered users in affected zones

- Mobile App Push Notifications: Real-time alerts with map links

- Dashboard: Web portal for emergency managers with live data feeds

- Siren Systems: Automated activation in high-risk communities

# 6. Data Flow Architecture

System Integration Diagram:

[Sensor Network] → [API Gateway] → [Time-Series DB] → [Data Processing] → [Physical Models] → [AI/ML Models] → [Visualization Engine] → [Alert System]

## 6.1 Technology Stack

Data Ingestion:

- API Framework: FastAPI (Python) / Node.js Express

- Message Queue: Apache Kafka for high-throughput sensor data

- Time-Series Database: InfluxDB / TimescaleDB (PostgreSQL extension)

Modeling Infrastructure:

- HEC-HMS/RAS: Windows Server VM with automation scripts (Python HEC-RAS Controller)

- SWMM: Storm Water Management Model for urban drainage

- ML Framework: TensorFlow / PyTorch for LSTM/Random Forest training

- GPU Cluster: NVIDIA A100 for deep learning inference

Visualization & Alerts:

- GIS Server: GeoServer / ArcGIS Server for map services

- Frontend: React.js + Mapbox GL JS for interactive maps

- Notification Service: Twilio (SMS), Firebase Cloud Messaging (push)

# 7. Integration Points & APIs

## 7.1 External Data Sources

- IMD (India Meteorological Department): Real-time rainfall forecasts via FTP

- CWC (Central Water Commission): River gauge data via SOAP API

- NASA GPM: Satellite precipitation (IMERG dataset) via Earth Data API

- Sentinel Hub: Satellite imagery for flood extent validation

## 7.2 Internal APIs

POST /api/v1/sensors/ingest

Endpoint to receive real-time sensor data (water level, rainfall, soil moisture).

GET /api/v1/models/run-forecast

Trigger HEC-HMS/RAS simulation with current sensor state. Returns job ID for async processing.

GET /api/v1/outputs/flood-extent/{job_id}

Retrieve flood extent GeoJSON for completed model run.

POST /api/v1/alerts/send

Dispatch SMS/push notifications to registered users based on alert level.

# 8. Appendix: Technical Specifications

## 8.1 Model Run Times

| Model | Computational Time |
| --- | --- |
| HEC-HMS (24hr simulation) | 2–5 minutes |
| HEC-RAS 2D (1000 sq km area) | 30–60 minutes (GPU accelerated) |
| LSTM Forecast (24hr ahead) | <1 second (inference) |

## 8.2 Data Storage Requirements

- Time-Series Sensor Data: ~50 GB/year (5-min intervals, 100 sensors)

- Model Outputs (rasters): ~200 GB/year (daily simulations)

- Satellite Imagery Archive: ~1 TB (historical flood events)

- ML Model Weights: ~5 GB (LSTM, Random Forest, XGBoost)

## 8.3 Accuracy Metrics (Validation)

- HEC-RAS Flood Extent: 85–90% F1-score vs. observed satellite imagery

- LSTM Forecast: RMSE <0.3m for 6-hour lead time, <0.8m for 24-hour lead time

- Random Forest Susceptibility: AUC-ROC >0.85 on test dataset

# 9. Key Performance Indicators (KPIs)

This section defines the KPIs used to measure system performance across three dimensions: model accuracy, operational efficiency, and real-world impact. Each KPI specifies the formula used for calculation and the authoritative reference source.

## 9.1 Model Accuracy KPIs

Quantify the predictive performance of physical and AI/ML models against observed/ground-truth data.

| KPI | Description | Formula | Reference Source |
| --- | --- | --- | --- |
| RMSE | Root Mean Square Error of flood stage forecast (m) | RMSE = √(Σ(yi − ŷi)² / n) | WMO Guidelines on Hydrological Forecasting (WMO-No. 168); CWC Flood Forecasting Manual |
| NSE | Nash-Sutcliffe Efficiency for hydrograph fit (range: −∞ to 1) | NSE = 1 − [Σ(Qobs − Qsim)² / Σ(Qobs − Q̄obs)²] | Nash & Sutcliffe (1970), J. of Hydrology; HEC-HMS Technical Reference Manual |
| F1-Score | Flood extent classification accuracy (precision/recall harmonic mean) | F1 = 2 × (Precision × Recall) / (Precision + Recall) | scikit-learn documentation; Copernicus Emergency Management Service validation protocols |
| AUC-ROC | Area under ROC curve for susceptibility classification | AUC = ∫₀¹ TPR(FPR) d(FPR) | Fawcett (2006), Pattern Recognition Letters; USGS Landslide Susceptibility guidelines |
| POD | Probability of Detection — % of actual flood events correctly forecast | POD = Hits / (Hits + Misses) | WMO Forecast Verification (WMO-No. 1023); NOAA NWS verification standards |
| FAR | False Alarm Ratio — % of warnings that did not result in floods | FAR = False Alarms / (Hits + False Alarms) | WMO Forecast Verification (WMO-No. 1023); ECMWF Verification Guide |

Target Thresholds:

- RMSE: <0.3 m (6-hr lead), <0.8 m (24-hr lead)

- NSE: >0.75 (acceptable), >0.85 (very good)

- F1-Score: >0.85 for flood extent classification

- AUC-ROC: >0.85 for susceptibility models

- POD: >0.90; FAR: <0.30

## 9.2 Operational KPIs

Measure the system's runtime performance, reliability, and responsiveness in production.

| KPI | Description | Formula | Reference Source |
| --- | --- | --- | --- |
| Forecast Lead Time | Time between alert issuance and flood onset | Lead Time = T_flood_onset − T_alert_issued | WMO Multi-Hazard Early Warning Checklist; UNDRR Sendai Framework Indicator G |
| System Uptime | Availability of monitoring & forecasting services | Uptime % = (Total Time − Downtime) / Total Time × 100 | ITIL v4 Service Management; ISO/IEC 20000-1:2018 |
| Alert Delivery Rate | % of alerts successfully delivered to registered users | Delivery Rate = Delivered / Dispatched × 100 | ITU-T E.804 SMS QoS; Common Alerting Protocol (CAP) OASIS standard |
| Data Ingestion Latency | Time from sensor reading to availability in DB | Latency = T_db_write − T_sensor_capture | OGC SensorThings API spec; InfluxDB performance benchmarks |
| Model Run Frequency | Number of forecast runs per day | Runs/day = Successful Runs in 24 hr window | HEC-RAS automation guidelines; ECMWF operational schedule |
| Sensor Data Quality | % of valid sensor readings after QA/QC | Quality % = Valid Readings / Total Readings × 100 | WMO-No. 8 Guide to Instruments & Methods of Observation (CIMO Guide) |

Target Thresholds:

- Forecast Lead Time: ≥6 hours (minimum), ≥24 hours (target)

- System Uptime: ≥99.5% (4-nines target during monsoon season)

- Alert Delivery Rate: ≥95% within 60 seconds of dispatch

- Data Ingestion Latency: <30 seconds end-to-end

- Sensor Data Quality: ≥98% valid readings

## 9.3 Impact KPIs (Lives & Assets Protected)

Measure the real-world humanitarian and economic value delivered by the system. These KPIs are reported to government stakeholders and align with the UN Sendai Framework for Disaster Risk Reduction (2015–2030).

| KPI | Description | Formula | Reference Source |
| --- | --- | --- | --- |
| Population Warned | Number of people receiving timely alerts in risk zones | Pop_Warned = Σ (Pop_zone_i × Delivery_Rate_i) for all zones in alert | Sendai Framework Indicator G-1; WorldPop methodology |
| Lives Saved (Est.) | Estimated lives saved via timely evacuation | Lives_Saved = Pop_Exposed × Baseline_Mortality_Rate × Evacuation_Effectiveness | Sendai Framework Indicator A-1; CRED EM-DAT mortality baselines |
| Assets Protected | Monetary value of buildings & infrastructure safeguarded | Assets_Protected = Σ (Asset_Value_i × Damage_Avoided_Factor_i) | Sendai Framework Indicators C-2 to C-6; HAZUS-MH loss estimation methodology |
| Economic Loss Avoided | Direct economic loss prevented (₹ crore) | Loss_Avoided = Expected_Loss_No_Warning − Actual_Loss_With_Warning | Sendai Framework Indicator C-1; World Bank GFDRR economic loss methodology |
| Critical Infra Protected | Hospitals, schools, roads safeguarded by early action | Count of critical facilities in alert zone with response time > lead time | Sendai Framework Indicator D-1; NDMA Critical Infrastructure Protection Guidelines (India) |
| Evacuation Compliance | % of warned population that evacuated successfully | Compliance % = Pop_Evacuated / Pop_Warned × 100 | Post-event surveys; NDMA Standard Operating Procedures for Floods |
| Benefit-Cost Ratio | Return on investment of early warning system | BCR = Σ (Annual Loss Avoided) / Σ (System Cost: CapEx + OpEx) | World Bank/GFDRR EWS cost-benefit guidance; Hallegatte (2012) |

Target Thresholds:

- Population Warned: ≥90% of exposed population within alert zones

- Evacuation Compliance: ≥70% of warned population

- Critical Infra Protected: 100% of hospitals & schools in alert zones notified

- Benefit-Cost Ratio: >4:1 (World Bank EWS benchmark)

## 9.4 KPI Reporting Cadence

- Real-time: Operational KPIs (uptime, latency, alert delivery) on monitoring dashboard

- Per Event: Accuracy KPIs (RMSE, F1, POD/FAR) computed for each flood event

- Monthly: Aggregated operational performance report to system administrators

- Annual: Impact KPIs (lives saved, assets protected, BCR) reported to government and Sendai monitor

— End of Document —

© 2026 Straive. All rights reserved.