LANDSLIDE HAZARD MONITORING & EARLY WARNING

AI/ML-Based Landslide Susceptibility & Prediction System

End-to-End Workflow Documentation

Complete Technical Process Flow

From Geospatial Data to Actionable Insights

© 2026 Straive. All rights reserved.

# Table of Contents

- System Overview

- Phase 1: Data Inputs & Collection

- Phase 2: Geospatial Modeling & Simulation

- Phase 3: AI/ML Enhancement (Future)

- Phase 4: Outputs & Alerts

- Data Flow Architecture

- Integration Points & APIs

- Appendix: Technical Specifications

- Key Performance Indicators (KPIs)

# 1. System Overview

The Landslide Susceptibility Mapping + AI/ML-Based Prediction system is a comprehensive landslide hazard monitoring and early warning platform that integrates real-time IoT sensor data, satellite-derived topography, soil moisture, rainfall observations, and predictive intelligence to deliver actionable landslide risk assessments and emergency response guidance.

## 1.1 System Architecture

The system operates through four integrated pillars:

- Data Inputs: IoT landslide sensors, satellite topography, soil moisture & rainfall datasets

- Geospatial Modeling: Slope stability, debris flow routing & rainfall-threshold analysis

- AI/ML Enhancement: SVM, XGBoost, InSAR & rainfall-threshold predictive models (Phase 2)

- Outputs & Alerts: Susceptibility zone maps, exposure analysis, and early warning notifications

## 1.2 Workflow Summary

Input → Process → Model → Enhance → Output → Act

# 2. Phase 1: Data Inputs & Collection

Data inputs form the foundation of the landslide prediction system. Multiple data sources are ingested, normalized, and validated before being fed into geospatial models.

## 2.1 Data Inputs Overview

| Component | What It Generates | Measurement Range | Classification |
| --- | --- | --- | --- |
| IoT Landslide Sensors | Real-time ground displacement | 0–500 mm displacement | Stable / Minor Movement / Critical Shift |
| SMAP Soil Moisture Data | Soil saturation levels | 0–100% moisture | Dry / Saturated / Critical Saturation |
| Rainfall Data | Precipitation intensity & accumulation | 0–300 mm/hr | Low / Moderate / Extreme Rainfall |
| Population ¦ Buildings ¦ Roads | Exposure & vulnerability layers | 1–50,000 people/sq km | Sparse / Dense / Highly Dense |
| Satellite Topography (UAV, DEM) | Slope, elevation, terrain morphology | 1–30 m spatial resolution | Flat / Moderate / Steep Terrain |
| Slope Data | Slope angle derivation | 0–90° slope | Stable / Vulnerable / Critical |
| Terrain Curvature Data | Concave/convex/flat terrain mapping | Concave / Convex / Flat | Stable / Erosion-Prone |

Process Flow:

- IoT extensometers/inclinometers installed at high-risk slope locations

- Data transmitted via LoRaWAN/cellular networks to central server

- Satellite ingestion: SMAP, Sentinel-1/2, ALOS PALSAR for terrain & moisture layers

- Time-series normalization: data standardized to 15-minute intervals

- Validation: outlier detection and quality control algorithms applied

- Storage: time-series database (InfluxDB/TimescaleDB) + spatial DB (PostGIS)

Data Format (JSON API Response Example):

{ "sensor_id": "LS_001", "timestamp": "2026-05-18T10:30:00Z", "displacement_mm": 12.4, "soil_moisture_pct": 78, "location": {"lat": 11.0168, "lon": 76.9558}, "status": "warning" }

Integration Requirement:

Geospatial modeling engines (ArcGIS Pro, QGIS, Scoops3D) require terrain rasters, soil moisture grids, and displacement time-series as boundary conditions for slope stability analysis.

# 3. Phase 2: Geospatial Modeling & Simulation

Physical geospatial models simulate the slope mechanics, debris flow behavior, and rainfall-triggered processes that govern landslide initiation. These deterministic models use limit-equilibrium and conservation equations to predict instability and runout.

## 3.1 Geospatial Modeling Components

| Component | What It Generates | Measurement Range | Classification |
| --- | --- | --- | --- |
| Slope Stability Analysis | Factor of Safety per slope unit | FoS: 0–3 | Stable >1.5, Critical <1 |
| Debris & Mudflow Routing | Dynamic runout path & impact zone | 0–50 km flow distance | Low / Moderate / Severe Impact |
| Rainfall-Threshold Trigger | Intensity-duration trigger analysis | 50–300 mm/day | Low / Moderate / High Trigger Risk |
| Slope Stability Index Mapping | Spatial stability classification | Stability index 0–100 | Stable / Vulnerable / Critical |
| Ground Displacement Detection | Surface movement detection | 0–500 mm movement | Stable / Active Movement / Failure Risk |
| Watershed Delineation | Basin & catchment boundary mapping | Basin-scale coverage | Small / Medium / Large Basin |
| DEM Derivation & Curvature | Slope, aspect, curvature derivation | 1–30 m resolution | Stable / Erosion-Prone |
| Susceptibility Mapping | Composite hazard score raster | Risk score 0–100 | Low / Medium / High Risk |
| Temporal Prediction Modeling | Lead-time forecast classification | 1–72 hr lead time | Advisory / Warning / Emergency |
| Terrain Instability Assessment | Multi-factor instability scoring | Multi-factor risk score | Stable / Vulnerable / Severe |

Model Details — Slope Stability Analysis:

- Software: Scoops3D / GeoStudio SLOPE/W / SHALSTAB

- Method: Limit-equilibrium (Bishop's method, Janbu, infinite slope)

- Input Required: DEM, slope angle, cohesion (c'), friction angle (φ'), unit weight (γ), pore pressure ratio (ru), root cohesion

- Process: Compute Factor of Safety (FoS) per slope cell; FoS <1 indicates failure risk

Workflow Steps:

- DEM Pre-Processing: derive slope, aspect, curvature, flow accumulation

- Watershed Delineation: identify catchment boundaries and flow paths

- Soil/Geology Layer Assignment: map cohesion, friction angle, unit weight by lithology

- Pore Pressure Computation: link soil moisture to pore pressure ratio (ru)

- FoS Calculation: per-cell limit equilibrium analysis

- Susceptibility Classification: bin FoS into Low / Medium / High / Severe zones

Output Format:

- GeoTIFF raster: Factor of Safety per grid cell

- Shapefile polygons: susceptibility zones (Low/Medium/High/Severe)

- Vector outputs for debris flow runout paths

## 3.2 Slope Pre-Processing

- DEM derivation, slope/aspect & curvature calculation

- Watershed delineation and catchment boundary analysis

- Topographic Wetness Index (TWI) and Stream Power Index (SPI) derivation

## 3.3 Rainfall-Threshold Trigger Analysis

Triggering thresholds are derived from intensity-duration (ID) relationships fitted to historical landslide events. The most widely used form is:

I = α × D^(−β)

Where I = rainfall intensity (mm/hr), D = duration (hr), and α, β are region-specific empirical coefficients calibrated against historical landslide records (Caine 1980; Guzzetti et al. 2008).

# 4. Phase 3: AI/ML Enhancement (Future Roadmap)

Phase 2 integration (planned for 3+ months post sensor data availability) will augment deterministic geospatial models with predictive intelligence using machine learning algorithms trained on historical landslide events.

## 4.1 SVM Landslide Susceptibility Mapping

Model Purpose:

Support Vector Machine (SVM) classifiers separate landslide-prone vs. stable terrain using high-dimensional geospatial feature spaces with non-linear kernels (RBF, polynomial).

Training Data Required:

- Historical landslide inventory polygons (typically 200+ events)

- Topographic derivatives (slope, aspect, curvature, TWI, SPI)

- Geological & soil maps, distance to faults/drainage

- Land cover, NDVI, rainfall climatology

Output Format:

GeoTIFF raster (0–1 susceptibility score) classified into Low / Medium / High / Severe zones.

## 4.2 XGBoost / Gradient Boosting Classification

Model Purpose:

XGBoost ensemble of decision trees handles non-linear interactions among conditioning factors and provides feature importance — useful for explainability to government stakeholders.

Model Architecture:

- Estimators: 300–800 trees, max depth 4–8

- Learning rate: 0.01–0.1 with early stopping on validation AUC

- Class balancing: SMOTE oversampling for rare landslide class

- Loss: binary log-loss / multi-class softmax depending on severity tiers

## 4.3 SAR Interferometry (InSAR) Displacement

Model Purpose:

Persistent Scatterer InSAR (PS-InSAR) and SBAS techniques detect sub-centimeter ground deformation over time using radar imagery (Sentinel-1, ALOS-2).

Output:

- Velocity maps (mm/year) per persistent scatterer point

- Time-series displacement plots per location

- Acceleration indicators flagging slopes entering tertiary creep

## 4.4 Rainfall-Threshold Prediction Model

Model Purpose:

Probabilistic ML model (Random Forest / Logistic Regression) combines rainfall forecasts, antecedent moisture, and slope susceptibility to estimate landslide probability within a forecast window.

Output Format:

Probability raster (0–1) at 1-hour intervals over the next 72 hours, with confidence intervals.

Integration Point:

ML probabilities fused with deterministic Factor of Safety maps via ensemble methods (Bayesian Model Averaging) to improve forecast accuracy and quantify uncertainty.

# 5. Phase 4: Outputs & Alerts (Pillar 1)

Model outputs are transformed into actionable decision-support products for emergency responders, government agencies, and affected communities.

## 5.1 Outputs & Alerts Overview

| Component | What It Generates | Measurement Range | Classification |
| --- | --- | --- | --- |
| Landslide Susceptibility Zone Map | Spatial risk classification | 0–100 risk index | Low / Medium / High / Severe |
| Ground Deformation Detection | Active slope movement tracking | 0–500 mm shift | Stable / Active / Critical |
| Population Exposure Analysis | Affected population estimation | 100 – millions | Low / Medium / Critical |
| Infrastructure Exposure Index | Affected assets count | 1–1000+ assets | Minimal / Major Impact |
| Evacuation Safe Zone Analysis | Safe zone radius mapping | 1–20 km radius | Safe / Restricted / Unsafe |
| Road Blockage Prediction | Route accessibility forecast | Open / Partial / Closed | Accessible / High Risk / Blocked |
| Debris Flow Impact Mapping | Runout zone delineation | <1–100+ sq km | Minor / Moderate / Severe |
| Early Warning Notifications | Multi-channel alerting | Seconds–minutes | Advisory / Watch / Warning / Emergency |
| Emergency Response Priority | Triage of response actions | Routine–Critical | Standard / High Priority / Immediate |
| Shelter Capacity Monitoring | Real-time shelter occupancy | 0–100% occupancy | Available / Near Full / Full |

Visualization Method:

- Web GIS Platform: ArcGIS Online / Mapbox GL JS / Leaflet

- Base Layer: OpenStreetMap or satellite imagery for context

- Susceptibility Symbology: green (low) → yellow → orange → red (severe)

- Interactive Features: click on polygon to view risk score, exposed population, infrastructure

Export Formats:

- Shapefile / GeoJSON for GIS integration

- KML/KMZ for Google Earth visualization

- Static PNG/PDF maps for emergency briefings

## 5.2 Population & Infrastructure Exposure Analysis

Process:

- Spatial Overlay: intersect susceptibility/runout zones with population density grid (WorldPop, 100m)

- Building Footprint Analysis: overlay OpenStreetMap buildings to identify affected structures

- Infrastructure Assessment: count roads, bridges, hospitals, schools within hazard zones

Output Dashboard Example:

- Total affected population: 75,000 people (Low / Medium / Critical)

- Residential buildings at risk: 6,200 structures

- Critical infrastructure: 4 hospitals, 22 schools, 80 km roads

- Economic Loss Estimate: ₹1,200 crore (based on asset valuation)

## 5.3 Early Warning Notifications

Alert Trigger Thresholds:

- Advisory: Susceptibility >0.5 + 24–72 hr forecast rainfall above threshold

- Watch: Susceptibility >0.7 + 12–24 hr lead time + moderate displacement

- Warning: Susceptibility >0.85 + 3–6 hr lead time + active movement detected

- Emergency: FoS <1 OR critical displacement rate exceeded — immediate action

Dissemination Channels:

- SMS/WhatsApp: targeted messages to registered users in affected zones

- Mobile App Push Notifications: real-time alerts with map links

- Dashboard: web portal for emergency managers with live data feeds

- Siren Systems: automated activation in high-risk communities

# 6. Data Flow Architecture

System Integration Diagram:

[IoT Sensor Network] → [Satellite Ingestion] → [API Gateway] → [Spatial/Time-Series DB] → [Geospatial Models] → [AI/ML Models] → [Visualization Engine] → [Alert System]

## 6.1 Technology Stack

Data Ingestion:

- API Framework: FastAPI (Python) / Node.js Express

- Message Queue: Apache Kafka for high-throughput sensor data

- Time-Series + Spatial DB: TimescaleDB + PostGIS extension

Modeling Infrastructure:

- Geospatial Engines: ArcGIS Pro / QGIS / Scoops3D / SHALSTAB

- InSAR Processing: SNAP / GMTSAR / ISCE2 for Sentinel-1 SBAS

- ML Framework: scikit-learn (SVM), XGBoost, TensorFlow/PyTorch

- GPU Cluster: NVIDIA A100 for deep learning inference

Visualization & Alerts:

- GIS Server: GeoServer / ArcGIS Server for map services

- Frontend: React.js + Mapbox GL JS for interactive maps

- Notification Service: Twilio (SMS), Firebase Cloud Messaging (push)

# 7. Integration Points & APIs

## 7.1 External Data Sources

- IMD (India Meteorological Department): real-time rainfall forecasts via FTP

- GSI (Geological Survey of India): geology, lithology & landslide inventory via portal

- NASA SMAP: soil moisture (L3/L4 products) via Earthdata API

- ESA Sentinel-1/2 (Copernicus): SAR & optical imagery via Sentinel Hub / Copernicus DataSpace

- USGS / NASA SRTM: 30m DEM via USGS EarthExplorer

## 7.2 Internal APIs

POST /api/v1/sensors/ingest

Endpoint to receive real-time sensor data (displacement, soil moisture, rainfall).

GET /api/v1/models/run-susceptibility

Trigger slope stability + ML susceptibility computation. Returns job ID for async processing.

GET /api/v1/outputs/susceptibility-map/{job_id}

Retrieve susceptibility GeoTIFF/GeoJSON for completed model run.

POST /api/v1/alerts/send

Dispatch SMS/push notifications to registered users based on alert level.

# 8. Appendix: Technical Specifications

## 8.1 Model Run Times

| Model | Computational Time |
| --- | --- |
| Slope Stability (Scoops3D, 1000 sq km) | 15–30 minutes |
| Debris Flow Routing (per event) | 10–20 minutes (GPU accelerated) |
| InSAR Time-Series (Sentinel-1 stack) | 4–8 hours (offline batch) |
| SVM / XGBoost Susceptibility Inference | <5 seconds (inference) |
| Rainfall-Threshold ML Forecast | <1 second (inference) |

## 8.2 Data Storage Requirements

- Time-Series Sensor Data: ~30 GB/year (15-min intervals, 100 sensors)

- Model Outputs (rasters): ~150 GB/year (daily susceptibility maps)

- Satellite Imagery Archive: ~2 TB (Sentinel-1 SAR stack + optical)

- ML Model Weights: ~3 GB (SVM, XGBoost, RF, deep models)

## 8.3 Accuracy Metrics (Validation)

- Slope Stability FoS: validated against historical failures with >80% match in failed zones

- SVM Susceptibility: AUC-ROC >0.85 on test dataset

- XGBoost Classification: AUC-ROC >0.88, F1-score >0.80

- InSAR Displacement: ±2 mm/year accuracy (Sentinel-1 PS-InSAR)

# 9. Key Performance Indicators (KPIs)

This section defines the KPIs used to measure system performance across three dimensions: model accuracy, operational efficiency, and real-world impact. Each KPI specifies the formula used for calculation and the authoritative reference source.

## 9.1 Model Accuracy KPIs

Quantify the predictive performance of geospatial and AI/ML models against observed/ground-truth landslide events.

| KPI | Description | Formula | Reference Source |
| --- | --- | --- | --- |
| AUC-ROC | Area under ROC curve for susceptibility classification | AUC = ∫₀¹ TPR(FPR) d(FPR) | Fawcett (2006), Pattern Recognition Letters; USGS Landslide Susceptibility guidelines |
| F1-Score | Landslide vs. non-landslide classification accuracy (precision/recall harmonic mean) | F1 = 2 × (Precision × Recall) / (Precision + Recall) | scikit-learn documentation; ISPRS landslide mapping validation protocols |
| Kappa Coefficient | Agreement between predicted and observed beyond chance | κ = (P_o − P_e) / (1 − P_e) | Cohen (1960); Landis & Koch (1977) Biometrics |
| TSS | True Skill Statistic (Hanssen-Kuipers Discriminant) | TSS = Sensitivity + Specificity − 1 | Allouche et al. (2006), J. of Applied Ecology |
| POD | Probability of Detection — % of actual landslides correctly forecast | POD = Hits / (Hits + Misses) | WMO Forecast Verification (WMO-No. 1023); GSI evaluation methods |
| FAR | False Alarm Ratio — % of warnings without landslide occurrence | FAR = False Alarms / (Hits + False Alarms) | WMO Forecast Verification (WMO-No. 1023); ECMWF Verification Guide |
| InSAR Velocity Accuracy | Detection accuracy of slope deformation rates | RMSE_v = √(Σ(v_obs − v_meas)² / n) | ESA Sentinel-1 InSAR Best Practices; Crosetto et al. (2016) ISPRS |

Target Thresholds:

- AUC-ROC: >0.85 (acceptable), >0.90 (excellent) for susceptibility models

- F1-Score: >0.80 for binary landslide classification

- Kappa: >0.60 (substantial agreement, Landis & Koch scale)

- TSS: >0.50 (good skill)

- POD: >0.85; FAR: <0.35

- InSAR Velocity RMSE: <2 mm/year (Sentinel-1 baseline)

## 9.2 Operational KPIs

Measure the system's runtime performance, reliability, and responsiveness in production.

| KPI | Description | Formula | Reference Source |
| --- | --- | --- | --- |
| Forecast Lead Time | Time between alert issuance and landslide event | Lead Time = T_event − T_alert_issued | WMO Multi-Hazard Early Warning Checklist; UNDRR Sendai Framework Indicator G |
| System Uptime | Availability of monitoring & forecasting services | Uptime % = (Total Time − Downtime) / Total Time × 100 | ITIL v4 Service Management; ISO/IEC 20000-1:2018 |
| Alert Delivery Rate | % of alerts successfully delivered to registered users | Delivery Rate = Delivered / Dispatched × 100 | ITU-T E.804 SMS QoS; Common Alerting Protocol (CAP) OASIS standard |
| Data Ingestion Latency | Time from sensor reading to availability in DB | Latency = T_db_write − T_sensor_capture | OGC SensorThings API spec; TimescaleDB benchmarks |
| Model Refresh Frequency | Number of susceptibility/forecast runs per day | Runs/day = Successful Runs in 24 hr window | USGS Operational Landslide Early Warning protocols |
| Sensor Data Quality | % of valid sensor readings after QA/QC | Quality % = Valid Readings / Total Readings × 100 | WMO-No. 8 Guide to Instruments & Methods of Observation (CIMO Guide) |
| InSAR Coherence | Average coherence quality of SAR interferograms | γ = ¦E[s₁s₂*]¦ / √(E[¦s₁¦²]·E[¦s₂¦²]) | ESA Sentinel-1 Toolbox documentation; Hanssen (2001) |

Target Thresholds:

- Forecast Lead Time: ≥6 hours (minimum), ≥24 hours (target)

- System Uptime: ≥99.5% (4-nines target during monsoon season)

- Alert Delivery Rate: ≥95% within 60 seconds of dispatch

- Data Ingestion Latency: <60 seconds end-to-end

- Sensor Data Quality: ≥98% valid readings

- InSAR Coherence: >0.5 across persistent scatterers

## 9.3 Impact KPIs (Lives & Assets Protected)

Measure the real-world humanitarian and economic value delivered by the system. These KPIs align with the UN Sendai Framework for Disaster Risk Reduction (2015–2030).

| KPI | Description | Formula | Reference Source |
| --- | --- | --- | --- |
| Population Warned | Number of people receiving timely alerts in risk zones | Pop_Warned = Σ (Pop_zone_i × Delivery_Rate_i) for all zones in alert | Sendai Framework Indicator G-1; WorldPop methodology |
| Lives Saved (Est.) | Estimated lives saved via timely evacuation | Lives_Saved = Pop_Exposed × Baseline_Mortality_Rate × Evacuation_Effectiveness | Sendai Framework Indicator A-1; CRED EM-DAT mortality baselines |
| Assets Protected | Monetary value of buildings & infrastructure safeguarded | Assets_Protected = Σ (Asset_Value_i × Damage_Avoided_Factor_i) | Sendai Framework Indicators C-2 to C-6; HAZUS-MH loss estimation |
| Economic Loss Avoided | Direct economic loss prevented (₹ crore) | Loss_Avoided = Expected_Loss_No_Warning − Actual_Loss_With_Warning | Sendai Framework Indicator C-1; World Bank GFDRR methodology |
| Critical Infra Protected | Hospitals, schools, roads safeguarded by early action | Count of critical facilities in alert zone with response time > lead time | Sendai Framework Indicator D-1; NDMA Critical Infrastructure Protection Guidelines (India) |
| Evacuation Compliance | % of warned population that evacuated successfully | Compliance % = Pop_Evacuated / Pop_Warned × 100 | Post-event surveys; NDMA Standard Operating Procedures for Landslides |
| Road Network Resilience | % of critical road km kept open during event | Resilience % = Open_km / Total_km × 100 | World Bank Resilient Infrastructure framework; MoRTH guidelines (India) |
| Benefit-Cost Ratio | Return on investment of early warning system | BCR = Σ (Annual Loss Avoided) / Σ (System Cost: CapEx + OpEx) | World Bank/GFDRR EWS cost-benefit guidance; Hallegatte (2012) |

Target Thresholds:

- Population Warned: ≥90% of exposed population within alert zones

- Evacuation Compliance: ≥70% of warned population

- Critical Infra Protected: 100% of hospitals & schools in alert zones notified

- Road Network Resilience: ≥80% of critical road km kept open

- Benefit-Cost Ratio: >4:1 (World Bank EWS benchmark)

## 9.4 KPI Reporting Cadence

- Real-time: Operational KPIs (uptime, latency, alert delivery) on monitoring dashboard

- Per Event: Accuracy KPIs (AUC, F1, POD/FAR) computed for each landslide event

- Monthly: Aggregated operational performance + InSAR coherence reports

- Annual: Impact KPIs (lives saved, assets protected, BCR) reported to government and Sendai monitor

— End of Document —

© 2026 Straive. All rights reserved.